
A Quick Introduction to the Orion TI-30XS Multiview Talking Scientific Calculator and How to Learn More
The Orion TI-30XS Multiview Talking Scientific Calculator is fully-accessible and handheld. It is ideal for all students, middle school through college, and beyond. APH partnered with Orbit Research and Texas Instruments to make this powerful calculator accessible, and it is now available from Orbit Research.
The Orion TI-30XS Multiview Talking Scientific Calculator
What comes in the box?

· Orion TI-30XS Multiview Talking Scientific Calculator

· Hard slide cover

· Hard snap cover

· Standard A male to Micro-B male USB Cable

· USB wall plug
· Earbuds

· USB flash drive containing documentation and software – User’s manual
· Quick-Start Guide in braille format

· Quick-Start Guide in print format
General Orientation to the TI-30XS Portion

· Gently pull the snap cover off and slide the calculator into the hard slide cover.

· Nearly the entire calculator is the standard TI-30XS but the solar screen area at the top of the calculator has been replaced with the Orion talking portion.

· The 41 TI-30XS keys are located directly below the screen.
· The basic keypad has approximately 9 rows and 5 columns. 
· The 1st and 2nd rows only have 3 buttons and the space for a 4th and 5th button are taken up by a four-key directional pad in the top-right corner.
· The four keys of the directional pad are the arrow keys for the TI-30XS.

· Turn the calculator on by pressing the “on” button located in the first column, bottom row. It has a raised outline on the bottom-left corner of the key.
· Turn it off by pressing the 2nd key (located in the first column, top row) followed by the “on” key.
General Orientation and working with the Orion portion

Above the screen on the face of the calculator are three keys in a horizontal region, replacing the solar panel of the original calculator. The keys are labeled A, S/L, and ↺R from left to right, which stand for Access, Silence/Learn, and Repeat. These three keys are the Orion keys. Each of the Orion keys has multiple functions depending on how they are pressed, tapped, long pressed, or used in combination with the Access key.

Access Key (A)
· Tap the Orion A key followed by an Access Mode key to get the modified functionality for that particular key. For example, Access followed by the “on” key gives you the battery status.
· Holding the A key with a long press enters Orion settings which allows you to change settings such as speed and/or volume using the arrows on the directional pad. Pressing and holding the A key again turns off Orion settings. Pressing a non-Orion settings key also exits the Orion settings.
Silence/Learn Key (S/L)

· Tapping the S/L key silences whatever is currently being spoken.

· Pressing the A key followed by the S/L key turns off access functionality until one of the Orion keys is pressed or the calculator is turned off and on again. This is a good mode to use when an instructor would like to use the calculator without speech.

· Holding the S/L key with a long press switches the calculator to Learn mode. Pressing the same key again with a long press lets you exit Learn mode.
Repeat Key (↺R)
· Tapping the ↺R key repeats the last spoken text.

· Pressing the A key followed by the ↺R key spells out the last spoken text.

· Press and hold the ↺R key to read the entire screen.
Working with the Orion TI-30XS
· The Orion TI-30XS is equipped with a Learn mode designed to assist you with locating keys and confirming their functionality. Learn mode can be used at any time. When the unit is in Learn mode, the calculator retains its previous status. For example, if you are in the middle of an operation and enter the Learn mode, the calculator operation is not interrupted and the display remains unchanged.

· To enter Learn mode, press and hold the middle Orion key (S/L) located at the top of the calculator and above the screen.

· To assist you with locating a particular key, during Learn mode each key and its primary function is announced when pressed. When used in combination with the 2nd key, the Orion TI-30XS announces the secondary function for the pressed key.

· To exit Learn mode, press and hold the middle Orion key (S/L). The calculator returns to the speech and operational mode at precisely the same point you were at prior to entering Learn mode, and it restores all the previously set preferences.

· Check out the keypad with help on.
· Leave help and work a few problems.
Other Important Things to Know

· Fully charge the Orion battery the first time you use the calculator. This should take about four hours.

· Check your battery status before any major test or assignment and make sure the calculator is fully charged.

· The ports at the top of the calculator are the earphone jack on the left and the USB Micro-B port on the right.
Resources: Learning More about this Calculator
To learn more about this calculator, access the helpful user guide located at:
Orion TI-30XS MultiViewTM Talking Scientific Calculator User's Guide
Susan A. Osterhaus
Page 1
7/6/2024

